EvoTime Issue Prompts — Open Work

Consolidated register of everything still open: audit issues not yet done, plus the defects found on hardware during the 31 July bring-up

| Repository | C:\Users\highj\Documents\Projects\EvoTime |
| --- | --- |
| Supersedes | issue-prompts.docx (31 July 2026, morning). Everything marked DONE there has been removed; §5 lists it for context. |
| Inspected at | working tree of 31 July 2026, night — after the first confirmed durable Slate install |
| Staged image | build/dfu/slate-dfu.zip, SHA-256 prefix E54859689936 (built, not yet flashed) |
| Scope | 13 prompts covering 14 open issues: 10 carried over from the audit register, 4 opened on hardware |
| Numbering | I-series and N-1…N-7 keep their original ids. N-8 onward were opened during hardware bring-up, continue the same sequence without colliding, and are cited by id in code comments — so they are not renumbered here. |
| Prompt target | Claude Code or an equivalent agent with repository write access |
| Status | Internal working document |

What this document is

One register of the work that is still open, replacing the morning audit document. Two things make it more than a filtered copy. First, several carried-over prompts had their premises invalidated by what the hardware showed tonight — I-3, I-9, I-10, I-13 and I-14 all contain instructions that are now wrong or already done. Those prompts have been rewritten, so do not work from the previous revision even for issues whose id and title look unchanged. Second, every item carries a Do not redo line naming the work that another issue already covers. §4 collects those overlaps in one place. The audit and the hardware findings reach the same code from different directions, and that is where duplicated effort would come from. Standing rules are unchanged. Below the SDP line — flash, DFU, BLE bring-up, boot, WDT/button, power — mirror InfiniTime; divergence needs a documented sealed-watch rationale. Wire constants live only in include/sdp_opcodes.hpp; base opcodes 0x01–0x61 and 0xF0–0xF1 are frozen; new primitives go in the length-prefixed 0xE0–0xEF range. RAM is capped at 54 KiB static+heap with ≥6 KiB slack.

# Contents

Contents	2

1. What changed since the previous revision	3

2. How to use this document	4

3. Where the hardware stands	5

4. Overlap map — what must not be worked twice	6

Part II — The prompts	7

A. In flight — verification and diagnosis	7

N-10 / N-11 — Verify the FreeRTOS tick fixes on hardware	7

N-12 — Confirm the battery reading and calibrate the curve	7

N-13 — App loop runs ~30x slower than designed	8

B. Firmware and protocol	9

I-3 — Tickless idle on RTC1: suppress-path review and soak	9

I-9 — InfiniTime low-level divergence audit	9

I-10 — FreeRTOS task topology split (design doc)	10

I-13 — Prove channel-5 SDP OTA end-to-end on hardware	10

I-14 — RAM headroom watch	11

I-11 — HRS3300 driver + Heart Rate Service (behind the flag)	11

I-12 — Current Time Service (CTS) for non-companion sync	12

C. Companion and ops	12

I-7 — Nordic DFU reports failure on a successful swap	13

I-8 — Harden sealed-DFU target classification	13

I-15 — Companion UX when the watch is not advertising	14

5. Suggested execution order	15

6. Closed since the previous revision	16

# 1. What changed since the previous revision

The morning document was written from a code audit, before any of it had run on the watch. Tonight the recovered PineTime went from a soft-brick to a confirmed, durable Slate install, and the act of getting there closed some issues and opened others.

Closed by code work: N-4 (frame reassembler now enforces single-in-flight across channels), N-6 (HELLO_OFFER encoder is capacity-checked with a build-time size guard and byte-exact goldens on both sides of the wire), and N-8 (the ARM image had stopped linking — the N-1 inbox pushed the heap into the stack reserve; the inbox is now zero-copy).

Closed by hardware: I-1 and I-2 completed — the watch was recovered, the fixed package flashed, and the amber trial bar cleared, so IMAGE_OK is written and the install survives reset. I-6 and I-16 were exercised for real in the process. N-9 (the radio never advertised: no identity address was configured, so ble_gap_adv_start returned BLE_HS_ENOADDR) is fixed and verified.

Opened by hardware: N-10 and N-11 (the FreeRTOS tick — no catch-up from the RTC counter, and an interrupt priority that was double-shifted to 0), N-12 (the battery ADC was configured at gain 1 instead of 1/4 and 10-bit under a constant named 12-bit, so every sample railed and the watch read 0 %), and N-13 (the app loop iterates roughly thirty times slower than designed). N-10, N-11 and N-12 have fixes staged and unverified; N-13 is instrumented but undiagnosed.

Three of those four are mirror-rule violations that the audit could not have caught by reading Slate alone — they are gaps against what InfiniTime does, in code that had simply never run. That is worth remembering when weighing further "improvements" below the SDP line.

# 2. How to use this document

Each prompt is self-contained: it names the files it touches, the behaviour it must not change, and its acceptance criteria. Run them one at a time and review the diff before the next. Where an item depends on another, the dependency is stated in its metadata rather than implied by ordering.

The sealed watch has no SWD, so the on-screen diagnostic overlay is the only debugger. Its format is reset/uptime_s/paints/button/worst_stall_ms/ble_state.rc/recovered_ticks on the first line and worst_phase.ms/adc_raw/battery_mV on the second. Reset reason is the raw RESETREAS bitmask and accumulates (2 = watchdog, 4 = soft). BLE state 7 means advertising; 90–95 are named bring-up failures with the return code appended (ble::bringup_snapshot, include/ble_gatt.hpp). Loop phases are 1 drain, 2 input, 3 session/core tick, 4 paint, 5 battery, 6 the notify wait — phase 6 dominating means the app task was starved rather than slow. Keep return codes captured; the two hardest bugs tonight were both found by a number that had previously been discarded.

A cyan screen is a configASSERT: the top hex word is FNV-1a of the source basename, the bottom is the line number. Decode with file_hash in src/boot_diag.cpp. Fatal paints deliberately stop petting the watchdog so the bootloader reverts to InfiniTime — a cyan screen followed by InfiniTime is the safety net working, not a second failure.

# 3. Where the hardware stands

The watch is running a confirmed Slate image and is stable for minutes at a time. BLE advertises as “Slate” at E8:01:34:22:08:89 and accepts connections. The companion connects, but ATT MTU stays at 23 with PHY and connection interval unnegotiated, service discovery does not complete, and the link drops periodically — all plausibly downstream of N-13.

The immediate next action is to flash `E54859689936` and read the overlay. It carries the N-10, N-11 and N-12 fixes plus the N-13 instrumentation, and one reading settles three open questions: whether the watchdog resets are gone (reset reason back to 4), whether the battery now reports a plausible voltage (second line, adc_raw/battery_mV), and which loop phase is consuming the iteration (second line, phase.ms). Until that is read, N-10/N-11, N-12 and N-13 are all provisional.

Recovery is proven and cheap: an unconfirmed image reverts on reset, and the button-hold reset was exercised successfully on hardware tonight. Flashing an experimental image is a low-risk act now in a way it was not this morning.

# 4. Overlap map — what must not be worked twice

These are the places where an audit issue and a hardware issue touch the same code. Each is also repeated in the affected prompt, but they are collected here because the failure mode is someone picking up an issue in isolation and redoing settled work.

I-5 → N-12. I-5 is closed, but its final caveat — “Slate’s adc * 2000 / 1241 conversion must be validated against a multimeter before reusing InfiniTime’s curve points” — turned out to be the whole of N-12. The conversion matched no working ADC configuration. Anyone reopening I-5 to chase the battery reading would rediscover N-12. The only remaining battery task is N-12’s multimeter check. I-3 → N-10, N-11. The old I-3 asked the agent to diff Slate’s tick ISR against the stock Nordic/InfiniTime port and decide whether to keep the Slate variant. That question is settled: the catch-up is restored with explicit guards (N-10) and the tick interrupt priority is fixed (N-11). I-3 now begins from that state and covers only the suppress path and the soak. I-10 → N-2, N-13. The old I-10 prompt states that the watchdog pet “currently lives in the tick + idle hooks”. That has been false since N-2 — the hooks do not pet at all, only the app loop does. A design doc written from the old premise would be wrong in its central safety argument. Separately, N-13’s remedy may turn out to be the topology split itself, so diagnose N-13 before designing I-10. I-14 → N-8. I-14’s budget figure of “+4.1 KiB for ble::AppInbox” is obsolete: the inbox borrows the reassembler buffer and costs about 12 bytes. The real and much tighter fact is that the heap now ends 496 B below the stack reserve. I-9 → N-9, N-10, N-11, N-12. Four mirror-gaps were found and fixed tonight. They belong in the parity table as settled entries; do not rediscover them as open questions. I-13 → N-12, N-13. SDP OTA gates on ≥30 % battery and treats unknown as blocked, so it cannot even start while the battery reads 0. A 120 KB transfer over a 23-byte MTU link that drops periodically would also test the wrong thing. Both are prerequisites, not excuses. I-7, I-8, I-15 share one scan/classifier path in the companion. They were separate issues in the audit but touch the same code; doing them in one pass avoids three conflicting implementations. I-15’s most common trigger — a watch that is silent — was N-9 and is fixed, so its scope shrinks to genuine cases.

# Part II — The prompts

## A. In flight — verification and diagnosis

### N-10 / N-11 — Verify the FreeRTOS tick fixes on hardware

| Source | Hardware bring-up, 31 July 2026 |
| --- | --- |
| Area | port/nrf52/src/port_rtc_tick.c |
| Status | Fix staged in E54859689936, unverified |
| Order | First — everything else reads the same clock |
| Do not redo | The stock-vs-Slate tick ISR comparison (settled here, not in I-3) |

Prompt

Two defects in the RTC1 tick are fixed in tree and need confirming on the watch. N-11 is the serious one. vPortSetupTimerInterrupt passed configKERNEL_INTERRUPT_PRIORITY to CMSIS NVIC_SetPriority. That macro is the already-shifted register form FreeRTOS writes straight into SHPR, while NVIC_SetPriority shifts internally, so the value was shifted twice and landed at 0 — the highest priority in the system, above NimBLE’s radio at 5 and above configMAX_SYSCALL_INTERRUPT_PRIORITY at 3. portENTER_CRITICAL raises BASEPRI to 3 and cannot mask priority 0, so xTaskIncrementTick could mutate the delayed-task lists inside another task’s critical section. It was latent since the port was written and never asserted because xTaskIncrementTick does not validate its caller. The fix passes the raw number plus a _Static_assert. N-10 restores InfiniTime’s catch-up: the handler now advances to wherever COUNTER actually is, guarded so a diff in the top half of the 24-bit range (tick ahead of counter — the original underflow-spin regression) steps once rather than ~16M, and at most 128 ticks are stepped per ISR. 1. Flash E54859689936 and let it run at least ten minutes with the companion connected. Record the overlay each minute. 2. Confirm: reset reason returns to 4 (no watchdog bit 2); uptime grows monotonically without the ~30 s reboot cycle; paints track roughly uptime ÷ 0.32 s; and the recovered-ticks field stays small. A large recovered-ticks value would mean the tick is still being held off and is worth investigating even though the catch-up is absorbing it. 3. Re-verify the watchdog still protects: hold the button and confirm the watch resets, proving the pet is still withheld while held. 4. Record the outcome in issues.md and add both defects to the I-9 parity table as aligned. Audit the tree for any other call passing a pre-shifted priority to a CMSIS setter — GPIOTE in src/cst816s.cpp runs at the default priority 0 and is safe only because it makes no FreeRTOS calls; note it rather than changing it blind. Acceptance: an hour of uptime with BLE connected, no watchdog resets, no cyan asserts, and the button-hold reset still working.

### N-12 — Confirm the battery reading and calibrate the curve

| Source | Hardware bring-up, 31 July 2026 (absorbs the open remainder of I-5) |
| --- | --- |
| Area | power.cpp::sample_battery_adc, battery.cpp::millivolts |
| Status | Fix staged in E54859689936, unverified |
| Order | Early — blocks I-13 |
| Do not redo | I-5 is closed; its unknown-state, curve, clamp and hysteresis work all shipped |

Prompt

The watch reported a valid 0 % on a charged cell. Two independent register bugs caused it, both now fixed in tree. The SAADC CH0_CONFIG GAIN field held 5, which is gain 1, not the 1/6 its comment claimed (encoding: 0 = 1/6 … 5 = 1). With the internal 0.6 V reference that puts full scale at 0.6 V while the divided battery pin sits near 2.0 V, so every sample railed. Separately RESOLUTION was written from a constant named RES_12BIT whose value was 1, which is 10-bit. The adc * 2000 / 1241 conversion matched neither configuration. The fix mirrors InfiniTime — gain 1/4 with the internal reference, 10-bit, mV = raw * 8 * 600 / 1024 — and the constants are renamed so both encodings are written down. 1. Flash and read the overlay’s second line, which ends /<adc_raw>/<battery_mV>. Compare the millivolts against a multimeter across the cell. This is the measurement I-5 asked for and never got. 2. If the reading is off, the raw count tells you which half is wrong: suspect the divider assumption if the error is a clean factor of two, the gain or reference if it is not. Fix the configuration and the conversion together — they are one unit, and CLAUDE.md now says so. 3. Once the voltage is trusted, re-check InfiniTime’s six-point curve {3500:0, 3616:3, 3723:22, 3776:48, 3979:79, 4180:100} against observed behaviour across a charge cycle, including the 99 % charging clamp and the direction hysteresis. Extend tests/host/test_battery.cpp with any breakpoint you change. 4. Confirm the charge-detect pin still reads correctly (P0.12 low means charging) and that the bolt appears beside the percentage. Acceptance: displayed percentage within a few points of the cell’s true state of charge across a full charge cycle, -- shown only before the first sample, and the DFU/OTA battery gates behaving sensibly at the 30 % boundary.

### N-13 — App loop runs ~30x slower than designed

| Source | Hardware bring-up, 31 July 2026 |
| --- | --- |
| Area | app_loop in src/main.cpp, FreeRTOS task balance |
| Status | Instrumented in E54859689936, undiagnosed |
| Order | High — likely blocks the companion link and I-13 |
| Do not redo | Do not start I-10’s topology design until this is diagnosed; the remedy may be that split |

Prompt

The loop iterates about every 610 ms against a 20 ms target — 27 paints in 264 s — with a worst app-task stall of 1631 ms. Touch is therefore polled under twice a second, so input feels sluggish, and session cadence is stretched. It is the most likely reason the companion sits at ATT MTU 23 with PHY and interval unnegotiated, never finishes service discovery, and drops the link. Do not assume BLE contention. The very first flashed image, whose radio never advertised, already showed 111 s for 10 paints — the same 690 ms per iteration. Whatever this is, it predates the radio working. 1. Read the overlay’s second line, <worst_phase>.<ms>. Phases: 1 drain, 2 input poll, 3 session/core tick, 4 paint, 5 battery/BAS, 6 the notify wait itself. Phase 6 dominating means the app task was ready but not scheduled — look at what outranks it: the NimBLE LL task and the FreeRTOS timer daemon both sit at priority 4 against the app task’s 2. Any other phase names slow work directly and should be profiled further with the same board::micros() pattern. 2. If it is scheduling, establish where the CPU goes before changing priorities. Enable configGENERATE_RUN_TIME_STATS against board::micros() in a debug build, or sample uxTaskGetSystemState, and report per-task share. Resist the temptation to simply raise the app task above the LL task: the radio has hard real-time deadlines and InfiniTime keeps its display work below the stack for that reason. 3. If it is slow work, the prime suspect is the repaint path — a face with a CLEAR marks the whole screen dirty, and with two 240×8 tiles that is thirty tile passes, each re-interpreting the display list. Measure one show_current() and, if that is the cost, reduce it by dirty-rect tracking on the diagnostic text rather than by painting less often. 4. Whatever the cause, re-measure afterwards and confirm the companion reaches ATT MTU 247 with the negotiated PHY and interval, and that the link stops dropping. If MTU negotiation is still stuck once the loop is healthy, split that out as its own issue against hook_mtu and the negotiation sequence in ble_nimble.cpp. Acceptance: loop iterations within about 2x of the 20 ms target, worst stall comfortably under 1 s, MTU 247 negotiated, and a link that survives ten minutes without dropping.

## B. Firmware and protocol

### I-3 — Tickless idle on RTC1: suppress-path review and soak

| Source | issues.md — Active / blocking |
| --- | --- |
| Area | port/nrf52/src/port_rtc_tick.c, config/FreeRTOSConfig.h |
| Order | Late — only after N-10/N-11 are verified |
| Do not redo | The tick-ISR catch-up decision (N-10) and the interrupt priority (N-11) are settled |

Prompt

configUSE_TICKLESS_IDLE is still 0. The original regression — tick ISR catch-up underflow, ISR spin, bootloader revert at ~7 s — is now understood and guarded, and the awake-path catch-up has been restored to match the stock Nordic/InfiniTime port. What remains is the suppress path and the soak. 1. Review vPortSuppressTicksAndSleep against the restored awake path for consistency. The two must agree about who owns the correction: the suppress path steps diff - 1 then calls xTaskIncrementTick once (the Nordic Q&A 63828 pattern), while the ISR now also corrects from COUNTER. Prove they cannot double-count on wake — in particular that a pending TICK event left over from the sleep window is consumed exactly once. This is the specific interaction that bricked the watch the first time. 2. Extend the host simulation of the suppress path (tests/host) with a mocked RTC register block covering: wake exactly on CC0, early wake by another IRQ, diff == 0, diff == 1, 24-bit COUNTER wrap, eTaskConfirmSleepModeStatus() == eAbortSleep, and the interaction with the ISR catch-up when both run in the same window. 3. Check the CC0 programming margin: nRF52 RTC COMPARE events can be missed when CC is set within ~2 counts of COUNTER, and configEXPECTED_IDLE_TIME_BEFORE_SLEEP is 2. Show that is sufficient or add an explicit guard. 4. Confirm configPOST_SLEEP_PROCESSING → slate_wdt_pet still feeds the dog across a sleep while remaining withheld during a button hold, and that the 1 s sleep cap holds. 5. Write the soak checklist into docs/freertos-tickless.md: sealed watch, ≥24 h — button hold resets while idle-sleeping, BLE connect/disconnect/reconnect stays healthy, wall-clock drift versus phone under 2 s/day, no cyan asserts, paints continue. Only after that passes may configUSE_TICKLESS_IDLE become 1. Do not flip it in this task.

### I-9 — InfiniTime low-level divergence audit

| Source | issues.md — Architecture |
| --- | --- |
| Area | Boot, DFU, BLE, power parity audit |
| Order | Ongoing — documentation only |
| Do not redo | N-9, N-10, N-11 and N-12 are diagnosed and fixed; record them, do not re-investigate |

Prompt

Produce and maintain docs/infinitime-parity.md: a table auditing every low-level behaviour against InfiniTime main, with columns {area, InfiniTime behaviour + file ref, Slate behaviour + file ref, verdict: aligned / mirror-gap / documented exception}. The default verdict below the SDP line is mirror, not decide. Seed it with tonight’s four findings, all of which were mirror-gaps in code that had never run, and all now aligned: identity address — nRF52 has no public address and the port never set the FICR-derived static-random identity, so advertising failed with BLE_HS_ENOADDR (N-9); tick catch-up — the ISR advanced one tick per event with no correction from COUNTER, which the stock port has always done (N-10); tick interrupt priority — a double-shifted priority put the tick at 0, above the radio and above the syscall threshold (N-11); battery ADC — gain and resolution disagreed with both the comment and the conversion (N-12). Then continue the audit, at minimum: advertising cadence and payload (InfiniTime uses fast 32–47 for ~30 s then slow 1636–1651, device name in the scan response, HRS uuid16 + DFU uuid128 in the payload, appearance 0xC2, and restarts advertising on disconnect and on failed connect); WDT setup bits and reload cadence; button enable and sense; boot/confirm semantics including the 10 s dwell as the one documented exception (link I-6); power — HRS3300 disabled at boot, SPI and NOR flash sleep with the display, guarded by bootloader version because old bootloaders cannot re-init sleeping NOR flash; CTS timing (link I-12); DFU service behaviour during transfer. For each mirror-gap verdict, file a follow-up in issues.md. Do not change code in this task.

### I-10 — FreeRTOS task topology split (design doc)

| Source | issues.md — Deferred |
| --- | --- |
| Area | Scheduler / roadmap §3.1 |
| Order | After N-13 is diagnosed |
| Do not redo | Stage 1 (link→app handoff) shipped as N-1; the WDT pet has moved since the audit |

Prompt

Roadmap §3.1’s display / link / sensors / system split is still deferred. Stage 1 is done: GATT RX only reassembles and publishes into ble::AppInbox, and the app task drains it, so no rendering happens on the NimBLE host task. Write docs/task-topology.md — design and migration plan, no code. Correct a stale premise before you start. The audit version of this prompt said the watchdog pet “currently lives in the tick + idle hooks”. That has been false since N-2: the tick and idle hooks do not pet at all, and only the app loop does (plus bounded flash helpers and the future tickless POST_SLEEP). The safety argument therefore runs the other way — a wedged app task must continue to starve the dog, exactly as InfiniTime does, and any new topology must preserve that. This was verified on hardware tonight. Wait for N-13. The app task is currently starved or slow by roughly 30x, and the remedy may be this split or may be a priority correction. Designing the topology before knowing which would risk building a large change around the wrong cause. 1. Reference topology: InfiniTime runs MAIN/SystemTask (350 words, prio 1, 10-deep byte queue), a DisplayApp task, HeartRateTask and the NimBLE tasks, with ISR-safe messaging via PushMessage choosing xQueueSend versus xQueueSendFromISR on SCB->ICSR VECTACTIVE. Adopt the same queue-of-enum pattern per task. 2. Constraints: ≤54 KiB static+heap with ≥6 KiB slack, and note that the current image already sits 496 B below the stack reserve, so the proposal must show where the RAM comes from. Every task documents its stack; high-water marks are checked in debug. 3. Deliverables: the proposed task set with priorities and stack budgets summing inside the RAM gate; which app_loop responsibilities move where; queue depths and message enums; migration order (extract link first, then display); and a rollback plan per stage, each of which must boot on sealed hardware. 4. State explicitly what does not change: WDT withhold semantics, RTC1 tick ownership, NimBLE task configuration.

### I-13 — Prove channel-5 SDP OTA end-to-end on hardware

| Source | issues.md — Deferred |
| --- | --- |
| Area | ota_xfer + companion OTA UI |
| Order | After N-12 and N-13 — both are hard prerequisites |
| Do not redo | Nothing; but do not attempt the hardware run early |

Prompt

ota_xfer (chunked, resumable, SHA-256 verified, 30 % battery gate) has never completed on a durable, confirmed install. The install now exists, so this is finally reachable — but not yet. Prerequisites, both blocking. The battery must read true (N-12): the transfer gates on ≥30 % and treats unknown as blocked, so at a reported 0 % it will refuse to start and look like a protocol failure. And the link must be stable at a negotiated MTU (N-13): a ~120 KB image over a 23-byte MTU link that drops periodically would exercise the resume paths by accident and prove nothing about the happy path. 1. Write docs/ota-verification.md with the test matrix: happy path (full transfer → MCUBoot swap → amber → confirm), resume after link drop at 25/50/75 %, resume after the phone app is killed, battery-gate refusal below 30 % and unknown-battery refusal, SHA mismatch rejection, and the IMAGE_OK interplay — OTA must refuse to start while the current image is unconfirmed. Verify that guard exists in ota_xfer.cpp/ota_slot.cpp and add it if missing. 2. Add instrumentation so failures are diagnosable on sealed hardware: transfer progress and last error through the diagnostic overlay and RTT, and a companion-side per-chunk log with offsets. 3. Extend the desktop tests with a full simulated transfer including forced resumption points and a corrupted chunk, so the on-watch run validates radio and flash behaviour rather than protocol logic. 4. Record results per matrix row; any failure becomes a new issue.

### I-14 — RAM headroom watch

| Source | issues.md — Watch |
| --- | --- |
| Area | Linker / CI RAM gate |
| Order | Ongoing |
| Do not redo | N-8 is resolved; the AppInbox figure in the audit version of this prompt is obsolete |

Prompt

The hard gate is static+heap ≤54 KiB with ≥6 KiB slack. Current state: the image links with `__heap_end__` just 496 B below `__StackLimit`. That margin is the real constraint, and it is thin enough that any static growth needs a map check before it is committed. Recent history worth knowing: the N-1 handoff originally added a 4 KiB copy slot, which pushed the heap into the stack reserve and broke the ARM link entirely (N-8) — the inbox now borrows the reassembler buffer and costs about 12 bytes. Ignore the “+4.1 KiB” figure in the previous revision of this prompt. 1. CI delta reporting: extend the RAM gate (SLATE_RAM_LENGTH in CMakeLists.txt plus whatever script enforces the rule — locate it first) to parse the production link map and emit a per-section breakdown (.data, .bss, heap, per-task stacks, NimBLE pools) against a stored baseline. Fail on >512 B unexplained growth even inside the gate; warn under 8 KiB slack; fail under 6 KiB. Add an explicit check on the __heap_end__ versus __StackLimit margin, since that is what actually broke. 2. Runtime watermarks: in debug builds log xPortGetMinimumEverFreeHeapSize() and each task’s uxTaskGetStackHighWaterMark() periodically over RTT, and expose the worst values in the diagnostic overlay so sealed soaks capture them. 3. Malloc-failure path: confirm vApplicationMallocFailedHook paints red and then lets the watchdog expire so a sealed watch reverts with the colour visible. This path was exercised for real tonight by the assert handler and works. 4. Document the NimBLE heap sizing rationale in docs/ble.md beside the existing mbuf maths.

### I-11 — HRS3300 driver + Heart Rate Service (behind the flag)

| Source | issues.md — Deferred |
| --- | --- |
| Area | BLE / sensors (HRS3300) |
| Order | Late — once bring-up is stable |
| Do not redo | Nothing outstanding elsewhere |

Prompt

include/ble_gatt.hpp has heart_rate = false pending a driver. Implement the driver and GATT plumbing, flag-off by default. 1. Driver (src/hrs3300.cpp, C++ class, no dynamic allocation after init): I2C address 0x44, registers per InfiniTime drivers/Hrs3300.cpp — ENABLE (0x01), PDRIVER (0x0C), RES (0x16), HGAIN (0x17); init then immediate Disable() (PDRIVER 0x00), exactly like InfiniTime’s boot. The chip powers on enabled and power.cpp currently sleeps it ad hoc — make the driver the single owner of that responsibility and redirect the existing call. All TWI operations go through twi:: with timeouts. 2. Measurement path: defer the PPG algorithm. Expose raw ALS/HRS readings and a stub bpm() returning invalid; document that InfiniTime’s Ppg.cpp DC/AGC algorithm is the follow-up. 3. GATT: standard HRS 0x180D with Heart Rate Measurement 0x2A37 notify, gated by the existing flag; advertise the uuid16 only when the flag is on. Respect SLATE_BLE_MAX_CONNECTIONS 1 and the mbuf budgets in docs/ble.md. 4. Tests: host-side unit tests for the register sequences (mock TWI) and the GATT payload encoding. 5. Report the RAM delta from the link map against the 496 B margin noted in I-14 before merging. Acceptance: flag off leaves binary behaviour unchanged except HRS sleep ownership; flag on makes the service enumerable with stub notifications; the sensor sleeps whenever the flag is off or nobody subscribes.

### I-12 — Current Time Service (CTS) for non-companion sync

| Source | issues.md — Deferred |
| --- | --- |
| Area | Time sync (GATT CTS) |
| Order | Late — once bring-up is stable |
| Do not redo | Nothing outstanding elsewhere |

Prompt

Time sync is companion-only today: CONTROL op 0x20 plus a u32 LE epoch. Add standard GATT CTS so Gadgetbridge or anything else can set the clock, mirroring InfiniTime. 1. Implement a CTS server (service 0x1805, Current Time characteristic 0x2A2B, read/write) in src/ble_gatt.cpp. InfiniTime’s CurrentTimeService accepts the exact-time-256 write (year u16 LE, month, day, hour, minute, second, day-of-week, fractions256, adjust reason) and sets its DateTime controller. Map writes onto wall_clock the same way CONTROL 0x20 does, validating every length and field and rejecting-and-resyncing per the BLE parser rule. 2. Optionally, as a second stage for parity, a CTS client that queries the central after connect with discovery deferred ~3 s like InfiniTime’s bleDiscoveryTimer, to avoid host/target discovery collisions. Note in code why the delay exists. 3. Precedence: companion CONTROL 0x20 wins over CTS when both fire (last-writer-wins is acceptable; document it). Keep the wire constant untouched in include/sdp_opcodes.hpp. 4. Host-side tests: a valid write sets the clock; short, oversized and out-of-range writes are rejected. 5. Verify the RAM delta against the gate.

## C. Companion and ops

These three share one scan-and-classify path. Doing them in a single pass is strongly preferred — separately they will produce three overlapping implementations of the same logic.

### I-7 — Nordic DFU reports failure on a successful swap

| Source | issues.md — Firmware UX |
| --- | --- |
| Area | Companion sealed DFU client |
| Order | Mid — with I-8 and I-15 |
| Do not redo | Shares the scan classifier with I-8 and I-15 |

Prompt

NordicLegacyDfuClient.kt already treats link loss during ACTIVATE as success, but operators still see failure toasts when the watch resets at other legitimate points after the image is fully transferred (GATT status 8, supervision timeout). Tighten the classification and verify after the fact. 1. Introduce an explicit transfer phase (CONNECTING / TRANSFERRING / IMAGE_COMPLETE / ACTIVATING). Any LinkLostException at IMAGE_COMPLETE or later is not a failure — report “image transferred; watch is swapping — verify on the watch”. Before IMAGE_COMPLETE, keep failing hard. An explicit DFU error response remains a failure in every phase. 2. Add post-DFU verification in SealedDfuService.kt: after the link drops post-IMAGE_COMPLETE, scan up to ~90 s for the device readvertising — either Slate’s service UUID or an InfiniTime advertiser — and upgrade the notification to “watch rebooted and is advertising”. MCUBoot swaps take tens of seconds; be generous and show progress. 3. Reword the remaining failure copy so it tells operators to check the watch face before trusting the phone. 4. Unit-test the phase classification with the existing event-channel fakes, simulating disconnects at each phase. Acceptance: a transfer that completes and then loses the link never shows a red failure state; genuine mid-transfer drops still do.

### I-8 — Harden sealed-DFU target classification

| Source | issues.md — Firmware UX |
| --- | --- |
| Area | Companion preflight |
| Order | Mid — with I-7 |
| Do not redo | Shares the scan classifier with I-7 and I-15 |

Prompt

SealedDfuPreflight.classify is a heuristic: MCUBoot has no BLE, so the phone cannot prove the bootloader layout. The residual risk is accepted; narrow it. 1. Positive identification before sending: when connected to the target, require the Nordic legacy DFU service (InfiniTime DfuService UUID 00001530-1212-EFDE-1523-785FEABCD123) and read Device Information Service strings if present. Classify as InfiniTime (DIS match or advertised name), InfiniTime-recovery, or unknown-legacy-DFU. Only the first two proceed normally; unknown requires a typed confirmation showing the device fingerprint. 2. Keep the package-side checks strict: NordicDfuPackage.kt continues to reject non-0x0052 device types, SoftDevice and bootloader payloads, and non-MCUBoot images. Add a check that the MCUBoot header’s img_size fits the InfiniTime primary-slot contract (475136 minus header and trailer) so an oversized image cannot be sent toward a smaller-slot bootloader. 3. Document the irreducible risk in docs/flash-sealed.md: a custom bootloader exposing the same DFU service with a different secondary map can still be bricked, and the typed-confirmation gate is the mitigation. 4. Unit-test the classifier verdicts including the new unknown → confirm path.

### I-15 — Companion UX when the watch is not advertising

| Source | issues.md — Companion / ops |
| --- | --- |
| Area | Companion CDM + link service |
| Order | Mid — with I-7 and I-8 |
| Do not redo | N-9 removed the most common cause; re-scope rather than building for a scenario that no longer dominates |

Prompt

CDM associate and “Start reconnect” spin forever against a silent radio. Note that the usual reason for that silence tonight was N-9, a firmware bug now fixed — so this is no longer the everyday case, but the diagnosis path is still worth having for genuine failures. 1. In the associate and reconnect flows (AssociationHelper.kt, LinkForegroundService.kt), add a bounded scan phase: if the target address or any Slate-service advertiser is not seen within ~30 s, stop and show a state page with three branches — the watch is on InfiniTime (detected by an InfiniTime-named or DFU-service advertiser) → offer the sealed-DFU flow; another app may hold the connection → run the existing LinkContention occupancy check; nothing heard at all → link to docs/recovery-sealed.md, including the discharge caveat. 2. startObservingDevicePresence stays for background reconnect, but foreground flows must not rely on it for feedback — CDM presence callbacks are silent when the radio is dead. Say so in a comment. 3. The sealed-installer entry point should reuse the same classifier so that selecting InfiniTime or recovery pre-checks that such an advertiser exists before opening the CDM chooser. 4. Unit-test the classifier on synthesised scan results: Slate advertiser, InfiniTime advertiser, silence.

# 5. Suggested execution order

Verify what is already staged first. Flash E54859689936, then N-10/N-11 → N-12 → N-13 in that order, because each reads the results of the one before: the clock has to be right before loop timings mean anything, and the loop has to be healthy before the link can be judged.

Then: I-7/I-8/I-15 as a single companion pass → I-13 once N-12 and N-13 clear → I-3 (soak) → I-10 once N-13 has named its cause → I-11 and I-12 last. I-9 and I-14 run alongside everything as documentation and CI work rather than as scheduled items.

# 6. Closed since the previous revision

Listed for context only — no action. Prompts for these are in the superseded document if the history is ever needed.

| I-1 | Sealed watch soft-brick — recovery runbook written, regression fenced by a host test, and the watch recovered in the field |
| --- | --- |
| I-2 | Fixed DFU package built, flashed and confirmed; the amber trial bar cleared and IMAGE_OK is written |
| I-4 | Local UI converted to scaled text; text_digits removed |
| I-5 | Battery unknown-state, InfiniTime curve, charge clamp and hysteresis shipped. Its open caveat became N-12 |
| I-6 | IMAGE_OK confirm — dwell kept as the documented exception, with a companion countdown; exercised for real tonight |
| I-16 | Multi-app BLE contention — LinkContention detection plus remediation copy; firmware resumes advertising on failed connect |
| N-1 | Display lists no longer parsed or rendered on the NimBLE host task; GATT RX publishes into ble::AppInbox and the app task drains |
| N-2 | Watchdog pet moved out of the tick and idle hooks into the app loop — verified on hardware: a button hold resets the watch |
| N-3 | now_ms() wrap fixed; monotonic source derived from the RTC counter, with regression tests across the old boundary |
| N-4 | Frame reassembler enforces single-in-flight across channels; companion SdpWriteQueue enqueues fragments atomically |
| N-5 | SPI transmit bounded with recovery and error propagation; always-on CS discipline checks |
| N-6 | HELLO_OFFER encoder capacity-checked with a build-time size guard and byte-exact goldens on both sides. Closed tonight — the previous document does not yet show it as done. |
| N-7 | Companion hardening: permission ceiling, redirect policy test, unknown-battery DFU gate |
| N-8 | ARM link restored — the handoff inbox is zero-copy; RAM back to 60,944 B |
| N-9 | Radio silent — no identity address was configured. Static-random identity derived from FICR; verified advertising on hardware |