EvoTime Issue Prompts — Open Work

Consolidated register of everything still open, after the 3 August session closed out bring-up: the watch now tells the time and updates itself over the air

| Repository | C:\Users\highj\Documents\Projects\EvoTime |
| --- | --- |
| Revision | Rev 3, 3 August 2026. Replaces rev 2 (1 August), which is stale — its headline item was I-13, since proven on hardware, and it predates N-19…N-26 entirely. |
| Running image | CFEDE90DE7F4 (0.1.0-m16). Installed by Slate’s own SDP OTA, confirmed IMAGE_OK. |
| Inspected at | working tree of 3 August 2026, after two successive Slate→Slate over-the-air updates |
| Scope | 8 prompts. I-13 and I-6 are closed; the display-list push is new and is the recommended next piece of work. |
| Live status | issues.md is the authoritative status register; this document holds the agent-ready prompts. Where the two disagree, issues.md wins. |
| Operator manual | docs/operator-manual.md describes every element on the watch and how to navigate between screens. |
| Prompt target | Claude Code or an equivalent agent with repository access and the build toolchain |

# Contents

Contents	2

1. What changed since rev 2	3

2. How to use this document	4

3. Where the hardware stands	5

4. Overlap map — what must not be worked twice	6

Part II — The prompts	6

A. The product thesis	6

P-1 — Push a display list from the phone and render it on the watch	6

B. Companion and ops	7

I-15 — Companion UX when the watch is not advertising	7

I-7 — Nordic DFU reports failure on a successful swap	8

I-8 — Harden sealed-DFU target classification	8

C. Firmware and architecture	9

I-9 — InfiniTime low-level divergence audit	9

P-8 — Flow control on the CONTROL channel	9

I-14 — RAM headroom watch	10

I-3 — Tickless idle on RTC1: suppress-path review and soak	11

5. Suggested execution order	12

6. Closed — no action	13

# 1. What changed since rev 2

Rev 2 described a watch that held a BLE link but had never negotiated an SDP session. That has been fixed, along with a chain of defects behind it. Bring-up is over.

The session had never negotiated. HELLO_OFFER is a notification, and it was being sent on the GAP connect event — roughly 1.3 s before the central writes the TX CCCD. NimBLE drops notifications to an unsubscribed client, so the offer went nowhere, every time, on every build ever flashed (N-23). The session now starts on BLE_GAP_EVENT_SUBSCRIBE.

The companion was connecting twice. connect() began with close(), and CDM’s presence callback fired ~250 ms after our own connect, tearing down a GATT that had requestMtu() outstanding. Its onMtuChanged never arrived, and because discovery is chained off that callback, services were never discovered and every send failed “not ready” (N-24).

Channel-5 SDP OTA is proven (I-13). Two consecutive Slate→Slate updates with no InfiniTime round-trip: 134180 B at ~46 kB/s, every 512 B chunk ACKed and credited, zero NAKs and zero resyncs, then reboot, swap and IMAGE_OK after the connected dwell. This is now the primary update path.

The clock works. Two defects: back-to-back CONTROL messages were being dropped by the single-slot inbox, so TIME_SYNC never arrived (N-25); and once it did, the companion was sending a UTC epoch to a watch with no timezone database (N-26). It now sends local wall-clock, as CTS does.

The watch is now self-describing. It shows its own version and build stamp, and live UPDATING nn% progress during a transfer. Several defects this session were mis-attributed because the only way to tell one image from another was to infer it from behaviour.

The companion has an in-app log (nRF Connect style) with copy and share, plus its version in the UI. Both of the last two root causes were found in that log.

# 2. How to use this document

Each prompt is self-contained: paste it verbatim. It states the goal, the constraints that must not be violated, and what “done” means. They assume the standing rules in CLAUDE.md — in particular that low-level behaviour mirrors InfiniTime and that SDP wire constants live only in include/sdp_opcodes.hpp.

Prompts are ordered by recommended sequence, not by id.

<w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:rPr><w:b/><w:bCs/></w:rPr><w:t xml:space="preserve">Before starting any prompt:</w:t></w:r><w:r><w:t xml:space="preserve"> read </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">issues.md</w:t></w:r><w:r><w:t xml:space="preserve"> first. It is updated continuously and this document is not. Eight of the defects fixed in the last two days were not in any prompt document at the time they were found.</w:t></w:r></w:p>

# 3. Where the hardware stands

The sealed watch boots reliably, holds a link at MTU 247, negotiates an SDP session, keeps time, reports true battery voltage, and updates itself over the air. Hold-to-blue rollback works even on a confirmed image and remains the guaranteed recovery path.

Verified on hardware: I-1, I-2, I-4, I-5, I-6, I-13, I-16, N-1 through N-26.

Not yet exercised at all: the remote UI path. Nothing has pushed a display list from the phone and seen it render. That is the product thesis and it is entirely unproven — which is why it is prompt 1 below.

Known thin margin: RAM sits at 60968 B of 64 KB (93%), about 490 B below __StackLimit. Any static growth needs a map check (I-14).

# 4. Overlap map — what must not be worked twice

I-12 (CTS) is effectively moot. TIME_SYNC on CONTROL now carries local wall-clock and syncs at 300 ms / 2 s / 8 s after Ready, then every 15 minutes. A CTS client would duplicate a working path. Close it unless non-companion sync becomes a real requirement.

I-7 and I-8 are demoted, not closed. Both concern Nordic legacy DFU, which is no longer the primary update path now that I-13 is proven. They still matter for recovery from a watch that cannot hold an SDP session.

I-9 overlaps everything. Nearly every defect in the N-series was a place Slate deviated from InfiniTime or from the stock NimBLE flow. Doing I-9 deliberately would likely retire several issues that have not been written down yet.

N-25 is fixed at the symptom, not the cause. The watch withholds CREDIT on a busy drop, but a write-without-response sender cannot act on it. Any future multi-message CONTROL burst has the same hazard. See prompt P-8.

# Part II — The prompts

## A. The product thesis

### P-1 — Push a display list from the phone and render it on the watch

| Status | Open — recommended next work |
| --- | --- |
| Area | Companion compositor ↔ firmware session / renderer |
| Depends on | Nothing. All prerequisites are proven. |
| Why now | Every defect fixed so far was plumbing. The transport is proven end-to-end; the payload has never been exercised. |

Prompt

<w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">The Slate link is now fully working: MTU 247, SDP session negotiated on subscribe, heartbeats flowing, time sync landing, firmware updating over channel 5. What has never been tested is the thing the whole architecture exists for — the companion composing a display list and the watch rendering it.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Goal: get one deliberately simple display list from the phone onto the watch screen, and one input event back.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">1. Establish what already flows. The companion log shows </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">ch=1</w:t></w:r><w:r><w:t xml:space="preserve"> (DISPLAY) and </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">ch=4</w:t></w:r><w:r><w:t xml:space="preserve"> (SYSTEM) traffic during a live session. Find out what those messages are, whether the watch parses them, and whether anything reaches the renderer. Report what you find before changing anything.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">2. Build the smallest end-to-end proof: a display list from the Kotlin DSL with a CLEAR, one filled rect and one text element, pushed on channel 1, taking over the screen from the local face.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">3. Verify on hardware with the diagnostic overlay: the parse and render millisecond fields on line 2 should move, and </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">remote_depth</w:t></w:r><w:r><w:t xml:space="preserve"> should become non-zero. Screen handover from local to remote must be visible.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">4. Send one touch event back. Wrap the rect in BEGIN_ELEM with EMIT_TOUCH and confirm the element id arrives at the companion.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">5. Then confirm the golden-file rule still holds: the Kotlin DSL and the JS builder must emit </w:t></w:r><w:r><w:rPr><w:b/><w:bCs/></w:rPr><w:t xml:space="preserve">byte-identical</w:t></w:r><w:r><w:t xml:space="preserve"> display lists. If no such test exists for this path, write it.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Constraints: the watch never executes anything received over BLE — display lists are data only. Every BLE-facing parser validates all lengths, indices and coordinates before use, and rejects and resyncs rather than faulting. Dirty-rect rendering is mandatory: a full-screen redraw is ~115 KB of SPI traffic.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Done when: a display list composed on the phone is visibly rendered on the watch, a tap on it is received by the phone, and the round-trip is described in </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">docs/</w:t></w:r><w:r><w:t xml:space="preserve"> with measured parse and render times.</w:t></w:r></w:p>

## B. Companion and ops

### I-15 — Companion UX when the watch is not advertising

| Status | Open — expected limitation, UX unclear |
| --- | --- |
| Area | Companion CDM + link service |
| Impact | A user with a silent watch has no path forward and no explanation |
| Priority | High — the last remaining way to strand a user |

Prompt

<w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">CompanionDeviceManager association and “Start / reconnect link” cannot find a radio that is not advertising. During a soft-brick, an InfiniTime-only recovery, or a watch sitting in the bootloader, both simply fail with no useful message. This was a real source of confusion during bring-up.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">1. Enumerate the states a watch can be in from the phone’s point of view: advertising as Slate, advertising as InfiniTime, advertising as the bootloader/recovery, connected to another central (Gadgetbridge, nRF Connect), and genuinely silent.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">2. Detect which one applies, using what is already available — a scan, </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">LinkContention</w:t></w:r><w:r><w:t xml:space="preserve"> instant GATT occupancy, and the CDM association record.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">3. Give each state a specific, actionable message. “Not found” is never sufficient. A watch held by Gadgetbridge needs different advice from one that is flat.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">4. Fold in the sealed-installer path: when the watch is only reachable as InfiniTime or recovery, steer to that flow rather than letting the user retry Slate association forever.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">5. Surface it in the existing Troubleshooting screen and the new in-app log rather than building a new UI surface.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Done when: each of the five states produces a distinct, correct explanation with a next action, verified by reproducing at least three of them on hardware.</w:t></w:r></w:p>

### I-7 — Nordic DFU reports failure on a successful swap

| Status | Open — cosmetic but misleading |
| --- | --- |
| Area | Companion DFU client / firmware reboot timing |
| Impact | A successful update looks like a failure |
| Priority | Medium — demoted now that SDP OTA is primary (I-13) |

Prompt

<w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">When a Nordic legacy DFU completes, the watch reboots to swap the image. The disconnect that follows is reported to the user as a failure even though the update succeeded. The same disconnect appears at the end of an SDP OTA as a </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">status=8</w:t></w:r><w:r><w:t xml:space="preserve"> supervision timeout, which the companion now handles correctly — use that as the model.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">1. Distinguish an expected post-transfer reboot from a genuine failure. The transfer having reached 100% and the last packet having been acknowledged is the discriminator.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">2. Report “installed, watch rebooting” and then wait for the reconnect, confirming success by reading the version or the confirm-status from the rebooted watch.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">3. Only report failure if the watch does not come back within a stated timeout, or comes back running the old image.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Done when: a successful Nordic DFU never shows an error, and a genuinely failed one still does.</w:t></w:r></w:p>

### I-8 — Harden sealed-DFU target classification

| Status | Open — heuristic |
| --- | --- |
| Area | Companion sealed-installer flow |
| Impact | Wrong target choice sends an image to the wrong device |
| Priority | Medium — demoted (see I-7) |

Prompt

<w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Deciding whether a discovered device is Slate, InfiniTime, the bootloader or something else is currently heuristic, based largely on advertised name. That is fragile: the same physical watch presents differently depending on what it is running.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">1. Classify on advertised service UUIDs rather than names where possible — Slate’s own service, the Nordic DFU service (0x1530) and InfiniTime’s characteristics are all discriminating.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">2. Handle the case where the identity address is the same across firmwares (it is — the FICR-derived address is stable, see N-9), so a cached classification can be stale after a flash.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">3. Refuse to start a transfer when classification is ambiguous, and say why, rather than guessing.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Done when: classification is UUID-driven, survives a firmware change on the same address, and ambiguity blocks rather than guesses.</w:t></w:r></w:p>

## C. Firmware and architecture

### I-9 — InfiniTime low-level divergence audit

| Status | Open — partially done, never completed systematically |
| --- | --- |
| Area | Firmware boot, BLE bring-up, peripherals |
| Impact | Unknown remaining divergences, each a latent defect |
| Priority | High — highest expected yield of anything on this list |

Prompt

<w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Nearly every defect in the N-series was a place where Slate deviated from InfiniTime or from the stock NimBLE flow: N-9 (identity address), N-11 (tick priority), N-12 (SAADC config), N-15 (who drives negotiation), N-16 (when services are registered), N-17 (UUID byte order), N-23 (when the session starts). Each was found by hitting it on hardware. The remaining ones are still there.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">CLAUDE.md</w:t></w:r><w:r><w:t xml:space="preserve"> states the standing rule: mirror InfiniTime for boot, MCUBoot/DFU, flash map, WDT/button and BLE radio bring-up; differ only above that, at SDP and companion level.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">1. Audit against these axes specifically, because they are the ones that actually produced defects: </w:t></w:r><w:r><w:rPr><w:b/><w:bCs/></w:rPr><w:t xml:space="preserve">initialisation ordering</w:t></w:r><w:r><w:t xml:space="preserve">, </w:t></w:r><w:r><w:rPr><w:b/><w:bCs/></w:rPr><w:t xml:space="preserve">who initiates</w:t></w:r><w:r><w:t xml:space="preserve"> an operation (watch or phone), </w:t></w:r><w:r><w:rPr><w:b/><w:bCs/></w:rPr><w:t xml:space="preserve">worst-case duration of work done inside a callback</w:t></w:r><w:r><w:t xml:space="preserve">, and </w:t></w:r><w:r><w:rPr><w:b/><w:bCs/></w:rPr><w:t xml:space="preserve">who writes peripheral ENABLE registers</w:t></w:r><w:r><w:t xml:space="preserve">.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">2. Cover: reset and clock bring-up, GPIO and pin configuration, SPI and TWI setup and teardown, the NimBLE host/controller task topology and priorities, GAP/GATT registration order, advertising parameters and resume policy, connection parameter negotiation, and the watchdog.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">3. For each divergence found, record: what InfiniTime does, what Slate does, whether it is deliberate, and the risk. Update </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">docs/infinitime-parity.md</w:t></w:r><w:r><w:t xml:space="preserve">, which currently has at least one row marked “aligned” that was not.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">4. Fix the ones that are accidental. Leave the deliberate ones documented with a reason.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Done when: </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">docs/infinitime-parity.md</w:t></w:r><w:r><w:t xml:space="preserve"> covers every axis above with no unexplained divergence, and every accidental one is either fixed or has a filed issue.</w:t></w:r></w:p>

### P-8 — Flow control on the CONTROL channel

| Status | Open — new, arising from N-25 |
| --- | --- |
| Area | Firmware ble_link / AppInbox, companion SdpWriteQueue |
| Impact | Any multi-message CONTROL burst can silently lose messages |
| Priority | Medium — mitigated but not solved |

Prompt

<w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">N-25 was fixed at the symptom. The underlying asymmetry remains: </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">AppInbox</w:t></w:r><w:r><w:t xml:space="preserve"> holds exactly one message and </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">ble_link</w:t></w:r><w:r><w:t xml:space="preserve"> gates all ingest while it is busy, withholding CREDIT. But the companion writes with </w:t></w:r><w:r><w:rPr><w:b/><w:bCs/></w:rPr><w:t xml:space="preserve">write-without-response</w:t></w:r><w:r><w:t xml:space="preserve">, so it never sees the withheld credit and nothing retransmits. Anything sent while the app task has not yet drained (a 20 ms window) is lost silently.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">The time sync was lost this way and the clock stayed at 1970 for entire sessions. It was only found because the clock was visibly wrong; a lost message with no visible symptom would not have been.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">1. Decide the model. Either the companion respects CREDIT on CONTROL as it already does on channel 5 (OTA), or CONTROL moves to write-with-response, or the inbox gains a small queue. State the trade-off in RAM (see I-14) and latency.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">2. Make drops observable regardless. </w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">note_busy_drop()</w:t></w:r><w:r><w:t xml:space="preserve"> already exists — surface the count in the diagnostic overlay and report it to the companion, so a silent loss stops being silent.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">3. Add a host test that bursts several CONTROL messages faster than the drain interval and asserts none are lost.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Done when: a CONTROL burst cannot silently lose a message, and the drop counter is visible on the watch and in the companion log.</w:t></w:r></w:p>

### I-14 — RAM headroom watch

| Status | Watch — 93% used, ~490 B below __StackLimit |
| --- | --- |
| Area | Linker / CI RAM gate |
| Impact | Static growth trips the gate or corrupts the stack |
| Priority | Medium — constrains every other prompt here |

Prompt

<w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">The hard gate is static + heap ≤ 54 KiB with ≥ 6 KiB slack. The current prod link map sits at 60968 B of 64 KB. The margin is thin enough that any new static allocation needs a map check — and P-1 (display lists) will want buffers.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">1. Produce a current breakdown by symbol: what actually occupies RAM, largest first. The NimBLE heap was bumped historically for the event queue; verify that is still the right size.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">2. Identify what can move to flash (const tables), what can be shrunk, and what is genuinely required.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">3. Make the CI gate report the top consumers on failure, so a regression names the culprit rather than just failing.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Done when: there is a documented breakdown, at least 6 KiB of genuine slack, and a CI failure that identifies what grew.</w:t></w:r></w:p>

### I-3 — Tickless idle on RTC1: suppress-path review and soak

| Status | Open — disabled, never soak-tested |
| --- | --- |
| Area | FreeRTOS port, port_rtc_tick.c, power |
| Impact | Battery life; risk of reintroducing N-10 style tick loss |
| Priority | Medium — now testable, since the watch stays up for hours |

Prompt

<w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Tickless idle is off. It interacts directly with the RTC1 tick port, which has already produced two serious defects: N-10 (tick loses time under BLE load, leading to a watchdog reboot loop) and N-11 (tick IRQ running at priority 0 because a raw priority was double-shifted).</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">1. Review the suppress path: how long sleep can be extended, how the catch-up is bounded (</w:t></w:r><w:r><w:rPr><w:rFonts w:ascii="Consolas" w:eastAsia="Consolas" w:hAnsi="Consolas" w:cs="Consolas"/><w:sz w:val="15"/><w:szCs w:val="15"/></w:rPr><w:t xml:space="preserve">kMaxCatchUp</w:t></w:r><w:r><w:t xml:space="preserve"> is 128 with a top-half-range guard), and what happens when the radio wakes the CPU mid-sleep.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">2. Confirm the WDT interaction. The tick and idle hooks deliberately do </w:t></w:r><w:r><w:rPr><w:b/><w:bCs/></w:rPr><w:t xml:space="preserve">not</w:t></w:r><w:r><w:t xml:space="preserve"> pet the watchdog — only the app task loop does. A tickless path that sleeps longer than the ~7 s dog without the app task running would reset the watch.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">3. Soak with the diagnostic overlay visible: the tick catch-up count (line 1, last field) must stay at 0, and worst phase 6 must not grow.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">4. Measure the actual current saving before deciding it is worth the risk.</w:t></w:r></w:p> <w:p><w:pPr><w:spacing w:after="120"/></w:pPr><w:r><w:t xml:space="preserve">Done when: a multi-hour soak shows zero tick catch-up and no watchdog resets, with a measured current figure justifying the change — or the item is closed as not worth it.</w:t></w:r></w:p>

# 5. Suggested execution order

1. P-1 — push a display list. Everything else is infrastructure for this, and it is the only item that tests the architecture rather than the plumbing.

2. I-15 — the last remaining way to strand a user with no explanation and no path forward.

3. I-9 — the divergence audit. Highest expected yield: it finds defects before hardware does, which is the opposite of how the last twenty-six were found.

4. P-8 and I-14 together — both constrain how P-1 can be built, and both are cheaper to address before more code depends on the current shape.

5. I-7 and I-8 — Nordic DFU polish, now a recovery path rather than the main one.

6. I-3 — tickless, only after a soak proves it is safe and a measurement proves it is worth it.

# 6. Closed — no action

I-13 — channel-5 SDP OTA. Proven on hardware 3 August, twice, with no InfiniTime round-trip. 134180 B at ~46 kB/s, zero NAKs, zero resyncs, IMAGE_OK confirmed after the connected dwell. The deliberate-failure rows of docs/ota-verification.md (mid-transfer disconnect, corrupt chunk on the wire, refusal while unconfirmed) remain proven only in host tests — worth running on hardware eventually, but not blocking.

I-6 — IMAGE_OK confirm vs single BLE connection. Resolved. The watch confirms after a dwell with a central connected, and the companion now logs the ON TRIAL → CONFIRMED transition explicitly.

I-12 — GATT CTS. Recommend closing. TIME_SYNC on CONTROL carries local wall-clock and works; a CTS client would duplicate it.

I-1, I-2, I-4, I-5, I-16 — closed in earlier sessions and verified on hardware since.

N-1 through N-26 — all closed. N-19 to N-26 are new since rev 2 and are documented in issues.md: OTA credit desync, whole-slot erase, bus mutex under an in-flight write, app task contention during transfer, session start on subscribe, duplicate connect, discovery watchdog, dropped CONTROL burst, and the UTC clock.